Slip-link code

Introduction

The code runs the slip-spring model originally developed by A. Lichtman. It was our intention to reproduce a variant of the model that has been utilized by our partners from Michelin (M. Couty, G. Maurel). It should be noted here that the code considers a pure polymer system. Some modifications should be made at later versions in order to be able to simulate systems of polymer blends and/or nanocomposites.

The model considers a system of non-interacting Rouse polymer chains (no excluded-volume interactions are included) of arbitrary length, which are constrained by additional springs representing the topological constraints called slip-links. At an initial stage, the code creates the slip-links and their corresponding anchoring points according to a gaussian distribution. Each SL is defined by a fixed anchoring point at position aj  and a ring attached to the chain at position sj . The number of SLs is defined at the start of the code and determines also the SL density. There is no restriction in the number of SLs attached to a given monomer. On the contrary, its anchoring point is allowed to be attached to no more than one SL.  The ring is constrained to move along the polymer chain by sliding along straight lines between neighboring monomers as:

sj=rtrunc(xj)+xj-truncxjrtruncxj+1-rtruncxj.                (1)

Here trunc(x) is the closest integer to x less than or equal to x and xj is the curvilinear abscissa of the ring along its chain. The anchoring point aj  is fixed in space provided that the SL is not destroyed. Each ring is connected to its anchoring point by a spring corresponding to the potential

USLsj=3kBT2Nsb2j=1Zaj-sj2,                                                          (2)

where 3kBT/2Nsb2 is the SL stiffness, counted in number of monomers  Ns , b being the average bond length, kBT the thermal energy and Z the total number of SLs attached to the chain. The total potential felt by a polymer chain having Z SLs is written as:

U=USL+UROUSE,                                                                     (3)

where

UROUSEri=3kBT2b2i=1Nmri-ri-12.                                              (4)

The motion of monomer i of a given chain obeys the Langevin equation as:

ξdridt=3kBTb2ri+1-2ri+ri-1+∇riUSL+fit,                           (5)

∇riUSL=3kBTNsb2j:truncxj=i1-xj-truncxjaj-sj+3kBTNsb2j:truncxj=i-1xj-truncxjaj-sj,                          (6)

fi(t) =0,                                                                   (7)

.
fi(t)fj(t') =2ξkBTδijδt-t'I,                                (8)

where ξ is the monomer friction coefficient. The force ∇riUSL on monomer i is due to the slip links between the monomers i-1 and i+1. Finally, fi(t) is a random Brownian force, satisfying the fluctuation/dissipation theorem (I stands for the unit tensor). The slip links obey a Langevin equation coupled to the previous one as:

ξsdxjdt=-∇xjUSL+gjt,                                           (9)

- ∇riUSL=3kBTNsb2rtruncxj+1-rtruncxjaj-sj,                         (10)

gi(t) =0,                                                        (11)

gi(t)gj(t') =2ξskBTδijδt-t',                                              (12)

where ξs is the slip link friction coefficient. The value of ξs is chosen to be much smaller than the monomer friction ξ, so that the diffusion of the SLs does not induce a significant additional dissipation.

Destruction/Recreation of SLs: We consider a static binary correspondence between pair SLs. When a SL passes through a chain end, i.e. when its monomer index is less than 0 or greater than n_m-1, it is destroyed and recreated at the extremities of any chain. At the same time, its companion is also destroyed and simultaneously recreated at a random position of a randomly chosen chain.

List of variables and functions :

Functions:

sl_init() : Performs the initial attribution and numbering of the SLs according to a Gaussian distribution.

force_rouse(): Calculates the force that acts on a monomer due to neighboring monomers. Monomers in a chain are connected by rouse springs.

force_sliplink(): Calculates the forces that act on chain monomers due to the sliplinks (if any).

move_beads(): Updates the monomer positions after the effect of the forces that act on the latter due to the neighboring monomers, due to the slip-links as well as due to the random forces.

move_sl(): Updates the positions of the SLs by integrating the equation of Langevin that they obey.

sl_analysis(): Performs static and dynamic analysis of the polymer chains. Calculated properties are: chain end-to-end distance, autocorrelation function of the chain end-to-end vector, autocorrelation function of the reorientation of the bond between two successive monomers, mean-squared displacement of the chain center of mass.

sl_destroy(): Checks if a SL or his pair have been destroyed and if yes, assigns a value of one to the corresponding flag variable (destroy) and maintains the binary correspondence between the pair of SLs.

sl_renew(): If one of the SLs of a SL pair has been destroyed, then it is recreated at the extremities of a randomly chosen chain, i.e. at the monomers indexed 0 or n_m-1. Its SL companion is also recreated simultaneously in a randomly chosen chain.

update_sl():

chain_init(): Creates an initial configuration of the polymer melt. Box dimensions are calculated in main() from the value of the density (variable rho) which is user-defined.

Random_number(): Gives a random number between 0 and 1.

Ran1(): Generates a random number between 0 and 1.

Variables

Global

in_ran1: Initiates the random number generator.

zzz: Total number of SLs.

n_ch: Total number of chains.

n_m:  Number of monomers per chain.

n_e:  Average number of monomers between SLs.

*destroy:  A pointer to each SL taking the value of 1 if the former has been destroyed or the value of 0 if it still exists.

maxstep:  Total number of simulation steps.

istep:  current step.

h1:  time step.

tauzero: Characteristic timescale.

output: Frequence (in time steps) of writing analysis data to output files.

h1: h*tauzero (timestep*characteristic time).

ksi: Friction constant of the chain monomers.

ksis: Friction constant of the SLs.

vol:  Volume of the simulation box.

b:  Average distance between monomers.

kBT:  The product (constant) of Boltzman’s constant times the temperature.

Rgir2:  Squared chain radius of gyration.

*box :  Simulation box size in each direction.

rho: System density (user-defined).

*endtoend: Table that contains the end-to-end distance of each chain.

*diff: Difference between the center-of-mass position of a chain at time t with respect to its position at istep=0, in each direction.

***Rb: Squared distance between two successive chain monomers. Calculated for each chain.

*P2: Autocorrelation function of the reorientation of a bond between two successive monomers (for each chain).

*MSD: Mean-squared displacement of the chain center-of-mass (for each chain).

*Ree2: Squared end-to-end-vector (for each chain).

*Dree: Autocorrelation function of the endtoend vector at time t (for each chain).

*Rbmean: Mean value of Rb (for each chain).

**Ree: Chain end-to-end vector (for each chain in each direction).

Rbmean_tot,P2_tot, Dree_tot, Ree2_tot, msd_tot, val_tot : Current step values of variables described earlier, summed over all chains.

***Rb0: Value of Rb at istep=0.

**Ree0: Ree at istep=0.

***XYZ:  Three-dimensional monomer position (the position calculated at the previous time step, except for the case at istep=0).

***XYZ0:  Three-dimensional monomer position, that is, the monomer position o fall monomers at istep=0, read by the file containing the initial configuration of the polymer melt.

**COM :  Chain center of mass in three dimensions.

**COM0 : Chain center of mass at istep=0;

***F_SP: Rouse force calculated at each direction (for each chain monomer and for each chain).

***F_SL: Sliplink force at each direction (for each monomer and every chain).

*chainj: the index of the chain a SL belongs to.

*xj: Curvelinear abscissa of a sliplink (for every sliplink).

*truncxj: The linear part of xj.

**XYZsj: SL coordinates at the current step.

**XYZaj: Coordinates of the anchoring point o SL (remain fixed, unless the SL is destroyed).

sigma:  Variance of random numbers, equal to the distance of anchoring points from monomers.

*ideb_SL: Integer denoting the index of the first SL belonging to a given chain.

*ifin_SL: Integer denoting the final SL of a given chain.

*num_SL : The total number of SLs of a given chain.

*SL: A pointer to a SL, used for numerotation of SLs.

main():

i,k,l,m,n:  counters.

*initfile, *end2endfile, *reefile, *p2file, *msdfile :  Pointers to output files as well as to a file containing the initial configuration of the polymer melt as created by chain_init().

In functions():

sl_init():

rand_idch_sl: Randomly selected chain index.

w1,lw1,w2,du1,u1: Variables used for the generation of anchoring points uniformly distributed around a SL.

k,m,q,j : counters.

: Random number multiplied that is by the total number of monomers (n_m) to give xj.

nc,nnc: chain index

sigma3 : A constant equal to (sqrt(Ns)*b)/sqrt(3.0).

sl_destroy():

ic: chain index.

ii,j,jj; counters for SLs.

sl_renew():

sigma3 : A constant described earlier.

rand_idch_sl: Randomly selected chain index.

w1,w2,lw1,lu1,u1: Variables used for the generation of anchoring points uniformly distributed around a SL.

nc_new, nnc_new: Indices for the randomly chosen chains of the recreated pair of SLs.

ic,ii,j,jj,k: counters.

txjj, txjjj : Indices of the monomer to which the recreated SLs are attached.

move_beads()

w1,w2,lw1,lu1,u1: Variables used for the generation of the random force that acts on a bead.

sigma1: A constant equal to sqrt(2.0*h*kBT/ksi), related to the random force acting on a monomer.

cte2 : A constant equal to 1/ksi.

i,j,l : counters.

Fsum: The sum of forces acting on a monomer divided by ksi (the friction parameter).

move_sl()

i,j,l : counters.

cte4: A constant equal to  3kBT/(Ns*b2*ksis).

sigma4: A constant equal to sqrt(2*h*kBT/ksis).

txjj: Index of the monomer to which the SL belongs.

xjtmp: Current value of the curvilinear abscissa of the SL.

force_rouse()

k,i: counters.

cte7: A constant equal to 3kBT/b2.

force_sliplink()

i,j,k,l: counters.

cte8: A constant equal to  cte7/ksis.

chain_init()

k,m: counters

w1,w2,lw1,du1,u1,grnd1,grnd2,grnd3: double numbers obtained randomly, used for the chain generation.

sl_analysis()

k,l,m,jj,j : counters

dree_tmp, msd_tmp, drb_tmp : Temporary values evaluated at the current step of the static and dynamic properties mentioned earlier.

update_sl()

k,j: counters.

txjj: Index of the monomer to which a SL is attached.

ic: chain index.